Reviewed by Bob Briscoe, 11 Aug 2014

Technical or normative points highlighted in yellow.

ConEx Working Group                                          S. Krishnan

Internet-Draft                                                  Ericsson

Intended status: Experimental                              M. Kuehlewind

Expires: August 18, 2014                     IKR University of Stuttgart

C. Ucendo

Telefonica

February 14, 2014

IPv6 Destination Option for ConEx

draft-ietf-conex-destopt-06

Abstract

ConEx is a mechanism by which senders inform the network about the

congestion encountered by packets earlier in the same flow.  This

document specifies an IPv6 destination option that is capable of

carrying ConEx markings in IPv6 datagrams.

Status of This Memo

This Internet-Draft is submitted in full conformance with the

provisions of BCP 78 and BCP 79.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF).  Note that other groups may also distribute

working documents as Internet-Drafts.  The list of current Internet-

Drafts is at http://datatracker.ietf.org/drafts/current/.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

This Internet-Draft will expire on August 18, 2014.

Copyright Notice

Copyright (c) 2014 IETF Trust and the persons identified as the

document authors.  All rights reserved.

This document is subject to BCP 78 and the IETF Trust's Legal

Provisions Relating to IETF Documents

(http://trustee.ietf.org/license-info) in effect on the date of

publication of this document.  Please review these documents

carefully, as they describe your rights and restrictions with respect

to this document.  Code Components extracted from this document must

include Simplified BSD License text as described in Section 4.e of

Krishnan, et al.         Expires August 18, 2014                [Page 1]

Internet-Draft          ConEx Destination Option           February 2014

the Trust Legal Provisions and are provided without warranty as

described in the Simplified BSD License.

Table of Contents

1.  Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   2

2.  Conventions used in this document . . . . . . . . . . . . . .   2

3.  Requirements for the coding of ConEx in IPv6  . . . . . . . .   2

4.  ConEx Destination Option (CDO)  . . . . . . . . . . . . . . .   3

5.  Implementation in the fast path of ConEx-aware routers  . . .   5

6.  Compatibility with use of IPsec . . . . . . . . . . . . . . .   6

7.  DDoS mitigation by using preferential drop  . . . . . . . . .   6

8.  Acknowledgements  . . . . . . . . . . . . . . . . . . . . . .   7

9.  Security Considerations . . . . . . . . . . . . . . . . . . .   7

10. IANA Considerations . . . . . . . . . . . . . . . . . . . . .   7

11. Normative References  . . . . . . . . . . . . . . . . . . . .   7

Authors' Addresses  . . . . . . . . . . . . . . . . . . . . . . .   8

1.  Introduction

ConEx [CAM] is a mechanism by which senders inform the network about

the congestion encountered by packets earlier in the same flow.  This

document specifies an IPv6 destination option [RFC2460] that ConEx-capable transport protocols can

use for performing ConEx marking in IPv6 datagrams.

This document solely specifies the ConEx wire protocol for use by any end-to-end transport protocol. Each transport protocol will need an update to specify precisely when a transport sets the various ConEx markings (e.g. the behaviour for TCP is specified in [ID.conex-tcp-modifications]).

The ConEx information can be used by any network element on the path

to e.g. do traffic management or egress policing.  Additionally this

information will potentially be used by an audit function that checks

the integrity of the sender's signaling.

1.1 Experiment Goals

Initially ConEx is specified experimentally so that the IETF can assess whether the compromises necessary in its design where the right ones.

Duration of Experiment: No less than 2 years from publication of this document will be needed to set up the infrastructure needed to determine the outcome of this experiment. Given ConEx is only chartered for IPv6, it might take longer to find a suitable test scenario where only IPv6 traffic is managed using ConEx.

Criteria for Success: The protocol will be deemed successful if, in the opinion of XXX? {the IETF’s Transport Area Director?}, it satisfies the requirements in [CAM], allowing it to be audited and trusted for use by traffic management functions, with minimal impact on performance.

2.  Conventions used in this document

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL","SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in [RFC2119].

3.  Requirements for the coding of ConEx in IPv6

[CAM] describes the ConEx mechanism in abstract terms and sets requirements for an ideal concrete protocol, but recognizes that it will be difficult to satisfy every requirement. This specification satisfies most of those requirements and gives justifications where compromises have had to be made.

The requirements in [CAM} relevant to wire protocol design are paraphrased below:

R-1: The marking mechanism needs to be visible to all ConEx-capable

nodes on the path.

R-2: The mechanism needs to be able to traverse nodes that do not

understand the markings.  This is required to ensure that ConEx can

be incrementally deployed over the Internet.

R-3: The presence of the marking mechanism should not significantly

alter the processing of the packet.  This is required to ensure that

Krishnan, et al.         Expires August 18, 2014                [Page 2]

Internet-Draft          ConEx Destination Option           February 2014

ConEx marked packets do not face any undue delays or drops due to a

badly chosen mechanism.

R-4: The markings should be immutable once set by the sender.  At the

very least, any tampering should be detectable.

R-5: The ConEx signals for packet loss and ECN marking SHOULD have

distinct encodings.

R-6: Additionally there SHOULD be an auditable ConEx Credit signal.

Based on these requirements four solutions to implement the ConEx

information in the IPv6 header have been investigated: hop-by-hop

options, destination options, using IPv6 header bits (from the flow

label), and new extension headers.  After evaluating the different

solutions, the wg concluded that the use of a destination option

would best address the requirements.

Choosing to use a destination option does not necessarily satisfy the requirement for on-path visibility, because it can be encapsulated by additional IP header(s). Therefore, ConEx policy or audit devices might have to bury into inner IP headers to find ConEx information. This choice was a compromise between fast-path performance and visibility, as discussed in Section 5.

4.  ConEx Destination Option (CDO)

The ConEx Destination Option (CDO) is a destination option that can

be included in IPv6 datagrams that are sent by ConEx-aware senders in

order to inform ConEx-aware nodes on the path about the congestion

encountered by packets in the same flow.  The CDO has an

alignment requirement of (none).

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Option Type  | Option Length |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|X|L|E|C|                       Reserved                        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 1: ConEx Destination Option Layout

Krishnan, et al.         Expires August 18, 2014                [Page 3]

Internet-Draft          ConEx Destination Option           February 2014

Option Type

8-bit identifier of the type of option. The option identifier

for the ConEx destination option will be allocated by the IANA.

Option Length

8-bit unsigned integer.  The length of the option (excluding

the Option Type and Option Length fields). This field MUST be

set to the value 4.

X Bit

When this bit is set, the transport sender is using ConEx with

this packet. If it is not set, the sender is not using ConEx with

this packet.

L Bit

When this bit is set, the transport sender has experienced a loss.

E Bit

When this bit is set, the transport sender has experienced

ECN-signaled congestion.

C Bit

When this bit is set, the transport sender is building up

congestion credit in the audit function.

Reserved

These bits are not used in the current specification. They

are set to zero on the sender and are ignored on the receiver.

All packets sent over a ConEx-capable connection MUST carry the CDO.

The CDO is immutable.  ConEx-aware functions read the flags, but all network devices MUST forward the CDO field unaltered.

IPSeC Authentication Header (AH) may be used to verify that the CDO

has not been modified.

Krishnan, et al.         Expires August 18, 2014                [Page 4]

Internet-Draft          ConEx Destination Option           February 2014

If the X bit is set, any of the other three bits (L, E, C) MAY be set.

Whenever one of these bits is set, the number of bytes carried by

this IP packet (including the IP header immediately preceding the CDO) SHOULD be accounted for

determining congestion or credit information.  In IPv6 the number of

bytes can easily be calculated by adding the number 40 (length of the

IPv6 header in bytes) to the value present in the Payload Length

field in the IPv6 header.

A transport sends credits prior to the occurrence of congestion (loss or

ECN-CE marks) and the amount of credits should cover the congestion

risk.  Note, the maximum congestion risk is that all packets in

flight get lost or ECN marked.

If the L or E bit is set, a congestion signal in the form of a loss or,

respectively, an ECN mark was previously experienced by the same

connection.

In principle all of these three bits (L, E, C) MAY be set in the same

packet.  In this case the packet size MUST be accounted more than

once for each respective ConEx information counter.

If a network node extracts ConEx information from a connection,

it is expected to hold this information in bytes,

e.g. comparing the total number of bytes sent with the number of

bytes sent with ConEx congestion marks (L, E) to determine the current

whole path congestion level.  For ConEx-aware node processing, the

CDO MUST use the Payload length field of the preceding IPv6 header

for byte-based accounting.  When a ratio is being measured and equally sized packets can be

assumed, counting the number of packets (instead of the number

of bytes) should deliver the same result.  But a network node must be

aware that this estimation can be quite wrong, if e.g. different

sized packed are sent, and thus it is not reliable.

The CDO is only applicable on unicast or anycast packets (see [CAM] for reasoning). A ConEx sender MUST NOT send a packet with X = 1 (ConEx-capable) to a multicast address, and it SHOULD NOT even include the CDO in such a packet. ConEx-capable network nodes MUST treat a multicast packet with the X flag set the same as an equivalent packet without the CDO. They MAY log the anomaly.

If the X bit is zero all the other three bits are undefined and thus

should be ignored by the receiver and forwarded unchanged by network nodes.  The X bit set to zero means that the connection

is ConEx-capable but this packet MUST NOT be counted when determining

ConEx information.  ConEx-capable network nodes MUST treat a packet with the X flag cleared the same as an equivalent packet without the ConEx destination option. They MAY log the anomaly.  This can be the case for

e.g. pure control packets not carrying any user data.  As an example

in TCP pure ACKs are usually not ECN-capable and TCP does not have a

mechanism to announce the loss of a pure ACK to the sender.  Thus

congestion information about ACKs are not available at the sender.

A ConEx-capable network node MUST forward unchanged any CDO with length other than 4, and treat the packet the same as it would treat an equivalent packet without CDO. It MAY log the anomaly.

A ConEx sender MUST set the reserved bits in the CDO to zero.

Other nodes MUST ignore these bits and ConEx-aware intermediate nodes MUST forward them unchanged, whatever their values. They MAY log the presence of a non-zero reserved field.

5.  Implementation in the fast path of ConEx-aware routers

ConEx information is being encoded into a destination option so

that it does not impact forwarding performance in non-ConEx-aware

nodes on the path.  Since destination options are not usually

processed by routers, the existence of the CDO does not affect

fast path processing of the datagram on non-ConEx-aware routers, i.e.

they are not pushed into the slow path for

exception processing.

Krishnan, et al.         Expires August 18, 2014                [Page 5]

Internet-Draft          ConEx Destination Option           February 2014

ConEx-aware nodes still need to process the CDO without severely

affecting forwarding.  For this to be possible, ConEx-aware

routers need to quickly ascertain the presence of the CDO and process

the option if it is present.  To efficiently perform this, the CDO

needs to be placed in a deterministic location.  In order to

facilitate forwarding on ConEx-aware routers, ConEx-aware senders that

send IPv6 datagrams with the CDO SHOULD place the CDO as the first

destination option in the destination options header. (This is not stated as a 'MUST', because some future destination option might need to be placed first for functional rather than just performance reasons.)

6. Configuration and Management

Once ConEx is implemented in a transport protocol it requires no configuration. For experimental purposes, it would be appropriate to be able to switch ConEx on or off at each sender, on a per-transport protocol basis.

There are no warning or error messages associated with the CDO.

7.  Compatibility with use of IPsec

In IPsec transport mode no action needs to be taken as the CDO is

visible to the network.  When accounting for ConEx information, the size

of the Authentication Header (AH) SHOULD NOT be accounted for as this

information has been added later.  In the IPsec Tunnel model the CDO

SHOULD be copied to the outer IP header as this information is end-

to-end.  Only the payload of the outer IP header minus the AH SHOULD

be counted.

If the transport network cannot be trusted, authentication SHOULD be

used to ensure integrity of the ConEx information.  If an attacker

would be able to remove the ConEx marks, this could cause an audit

device to penalize the respective connection, while the sender cannot

easily detect that ConEx information is missing.

It might be possible to implement a proxy for a ConEx sender, as long as it is located where receiver feedback is always visible. A ConEx proxy MUST NOT introduce a CDO header into a packet already carrying one and it MUST NOT alter the information in any existing CDO header. However, it can add a CDO header to any packets without one, taking care not to disrupt any integrity or authentication mechanisms.

8. Tunnel Processing

As with any destination option, an ingress tunnel endpoint SHOULD NOT copy the CDO when adding an encapsulating outer IP header. However, it MAY copy the CDO to the outer in order to facilitate visibility by subsequent on-path ConEx functions. This trades off the performance of ConEx functions against that of tunnel processing.

An egress tunnel endpoint SHOULD ignore any CDO on decapsulation of an outer IP header. The information in any inner CDO will always be considered correct, even if it differs from any outer CDO. Therefore, the decapsulator can strip the outer CDO without comparison to the inner. A decapsulator MAY compare the two, and MAY log any case where they differ. However, the packet MUST be forwarded irrespective of any such anomaly, given an outer CDO is only a performance optimisation.

9.  Mitigating flooding attacks by using preferential drop

This section is aspirational, and not critical to the use of ConEx for more general traffic management.

The only nodes other than the sender that need to parse the CDO are ConEx-aware policy or audit devices. However, once CDO information is present, the CDO header could optionally also be used in the data plane of any IP-aware forwarding node to mitigate flooding attacks.

If a router queue experiences very high load so that it has to drop

arriving packets, it MAY preferentially drop packets within the same

Diffserv PHB using the preference order given in Table 1 (1 means

drop first).  Additionally, if a router implements preferential drop

it SHOULD also support ECN-marking.  Preferential dropping can be

difficult to implement on some hardware, but if feasible it would

discriminate against attack traffic if done as part of the overall

policing framework as described in [RFC6789].  If nowhere else,

routers at the egress of a network SHOULD implement preferential drop

(stronger than the MAY above).

+----------------------+----------------+

|                      |   Preference   |

+----------------------+----------------+

| Not-ConEx or no CDO  | 1 (drop first) |

| X (but not L,E or C) |       2        |

| X and L,E or C       |       3        |

+----------------------+----------------+

Table 1: Drop preference for ConEx packets

Krishnan, et al.         Expires August 18, 2014                [Page 6]

Internet-Draft          ConEx Destination Option           February 2014

A flooding attack is inherently about congestion of a resource.  As

load focuses on a victim, upstream queues grow, requiring honest

sources to pre-load packets with a higher fraction of ConEx-marks.

If ECN marking is supported by downstream queues, preferential

dropping provides the most benefits because, if the queue is so

congested that it drops traffic, it will be CE-marking 100% of any

forwarded traffic.  Honest sources will therefore be sending 100%

ConEx E-marked packets (subject to rate-limiting at an

ingress policer).  Senders under malicious control can either do the

same as honest sources, and be rate-limited at ingress, or they can

understate congestion and not set the E flag.  If the preferential drop ranking is

implemented on queues, these queues will preserve E/L-marked traffic

until last.  So, the traffic from malicious sources will all be

automatically dropped first.  Either way, malicious sources

cannot send more than honest sources.

8.  Acknowledgements

The authors would like to thank Marcelo Bagnulo, Bob Briscoe, Ingemar

Johansson, Joel Halpern and John Leslie for the discussions that led

to this document.

Special thanks to Bob Briscoe who contributed text and analysis work

on preferential dropping.

9.  Security Considerations

This document does not bring up any new security issues concerning the overall ConEx integrity and traffic management framework that have not already been discussed in [CAM] In particular, [CAM] introduces the audit framework necessary to ensure that a sender has to set the CDO flags to reflect actual congestion experienced.

All undefined and invalid combinations of fields, including reserved fields, have been identified in Section 4, and appropriate behaviours have been assigned.

Protocol interaction with IPsec is addressed in Section 6.

Section 8 on using ConEx against flooding attacks is aspirational. If this proves not to be effective, it does not invalidate the use of ConEx for more general traffic management.

10.  IANA Considerations

This document defines a new IPv6 ConEx destination option for carrying

ConEx markings.  IANA is requested to assign a new destination option

type in the Destination Options registry maintained at http://

www.iana.org/assignments/ipv6-parameters <TBA1> ConEx Destination

Option [RFCXXXX] The act bits for this option need to be 00 and the

chg bit needs to be 0.

11.  Normative References

[CAM]      Mathis, M. and B. Briscoe, "Congestion Exposure (ConEx)

Concepts and Abstract Mechanism", draft-ietf-ConEx-

abstract-mech-05 (work in progress), July 2011.

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119, March 1997.

Krishnan, et al.         Expires August 18, 2014                [Page 7]

Internet-Draft          ConEx Destination Option           February 2014

[RFC2460]  Deering, S. and R. Hinden, "Internet Protocol, Version 6

(IPv6) Specification", RFC 2460, December 1998.

[RFC6789]  Briscoe, B., Woundy, R., and A. Cooper, "Congestion

Exposure (ConEx) Concepts and Use Cases", RFC 6789,

December 2012.

[ID.conex-tcp-modifications] draft-ietf-conex-tcp-modifications...

Authors' Addresses

Suresh Krishnan

Ericsson

8400 Blvd Decarie

Town of Mount Royal, Quebec

Canada

Email: suresh.krishnan@ericsson.com

Mirja Kuehlewind

IKR University of Stuttgart

Email: mirja.kuehlewind@ikr.uni-stuttgart.de

Carlos Ralli Ucendo

Telefonica

Email: ralli@tid.es

Krishnan, et al.         Expires August 18, 2014                [Page 8]