Congestion and Pre-Congestion                                 B. Briscoe

Notification                                                          BT

Internet-Draft                                              T. Moncaster

Obsoletes: 5696 (if approved)              Moncaster Internet Consulting

Intended status: Standards Track                                M. Menth

Expires: January 31, 2012                        University of Tuebingen

July 30, 2011

Encoding 3 PCN-States in the IP header using a single DSCP

draft-ietf-pcn-3-in-1-encoding-07

Abstract

The objective of Pre-Congestion Notification (PCN) is to protect the

quality of service (QoS) of inelastic flows within a Diffserv domain.

The overall rate of the PCN-traffic is metered on every link in the

PCN domain, and PCN-packets are appropriately marked when certain

configured rates are exceeded.  Egress nodes pass information about

these PCN-marks to decision points which then decide whether to admit

or block new flow requests or to terminate some already-admitted

flows during serious pre-congestion.

This document specifies how PCN-marks are to be encoded into the IP

header by re-using the Explicit Congestion Notification (ECN)

codepoints within a PCN-domain.  This encoding provides for up to

three different PCN marking states using a single DSCP: not-marked

(NM), threshold-marked (ThM) and excess-traffic-marked (ETM).  Hence,

it is called the 3-in-1 PCN encoding.  This document obsoletes

RFC5696.

Status of this Memo

This Internet-Draft is submitted in full conformance with the

provisions of BCP 78 and BCP 79.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF).  Note that other groups may also distribute

working documents as Internet-Drafts.  The list of current Internet-

Drafts is at http://datatracker.ietf.org/drafts/current/.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

This Internet-Draft will expire on January 31, 2012.

Briscoe, et al.         Expires January 31, 2012                [Page 1]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

Copyright Notice

Copyright (c) 2011 IETF Trust and the persons identified as the

document authors.  All rights reserved.

This document is subject to BCP 78 and the IETF Trust's Legal

Provisions Relating to IETF Documents

(http://trustee.ietf.org/license-info) in effect on the date of

publication of this document.  Please review these documents

carefully, as they describe your rights and restrictions with respect

to this document.  Code Components extracted from this document must

include Simplified BSD License text as described in Section 4.e of

the Trust Legal Provisions and are provided without warranty as

described in the Simplified BSD License.

Briscoe, et al.         Expires January 31, 2012                [Page 2]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

Table of Contents

1.  Introduction . . . . . . . . . . . . . . . . . . . . . . . . .  4

1.1.  Requirements Language  . . . . . . . . . . . . . . . . . .  5

1.2.  Changes in This Version (to be removed by RFC Editor)  . .  5

2.  Definitions and Abbreviations  . . . . . . . . . . . . . . . .  7

2.1.  Terminology  . . . . . . . . . . . . . . . . . . . . . . .  7

2.2.  List of Abbreviations  . . . . . . . . . . . . . . . . . .  8

3.  Definition of 3-in-1 PCN Encoding  . . . . . . . . . . . . . .  8

4.  Requirements for and Applicability of 3-in-1 PCN Encoding  . .  9

4.1.  PCN Requirements . . . . . . . . . . . . . . . . . . . . .  9

4.2.  Requirements Imposed by Tunnelling . . . . . . . . . . . . 10

4.3.  Applicability of 3-in-1 PCN Encoding . . . . . . . . . . . 10

5.  Behaviour of a PCN-node to Comply with the 3-in-1 PCN

Encoding . . . . . . . . . . . . . . . . . . . . . . . . . . . 11

5.1.  PCN-ingress Node Behaviour . . . . . . . . . . . . . . . . 11

5.2.  PCN-interior Node Behaviour  . . . . . . . . . . . . . . . 11

5.2.1.  Behaviour Common to all PCN-interior Nodes . . . . . . 11

5.2.2.  Behaviour of PCN-interior Nodes Using Two

PCN-markings . . . . . . . . . . . . . . . . . . . . . 12

5.2.3.  Behaviour of PCN-interior Nodes Using One

PCN-marking  . . . . . . . . . . . . . . . . . . . . . 12

5.3.  Behaviour of PCN-egress Nodes  . . . . . . . . . . . . . . 13

6.  Backward Compatibility . . . . . . . . . . . . . . . . . . . . 13

6.1.  Backward Compatibility with ECN  . . . . . . . . . . . . . 13

6.2.  Backward Compatibility with the Baseline Encoding  . . . . 14

7.  IANA Considerations  . . . . . . . . . . . . . . . . . . . . . 14

8.  Security Considerations  . . . . . . . . . . . . . . . . . . . 14

9.  Conclusions  . . . . . . . . . . . . . . . . . . . . . . . . . 15

10. Acknowledgements . . . . . . . . . . . . . . . . . . . . . . . 15

11. Comments Solicited . . . . . . . . . . . . . . . . . . . . . . 15

12. References . . . . . . . . . . . . . . . . . . . . . . . . . . 15

12.1. Normative References . . . . . . . . . . . . . . . . . . . 15

12.2. Informative References . . . . . . . . . . . . . . . . . . 16

Appendix A.  Choice of Suitable DSCPs  . . . . . . . . . . . . . . 17

Appendix B.  Co-existence of ECN and PCN . . . . . . . . . . . . . 18

Appendix C.  Example Mapping between Encoding of PCN-Marks in

IP and in MPLS Shim Headers . . . . . . . . . . . . . 20

Appendix D.  Rationale for Discrepancy Between the Schemes

using One PCN-Marking . . . . . . . . . . . . . . . . 22

Authors' Addresses . . . . . . . . . . . . . . . . . . . . . . . . 22

Briscoe, et al.         Expires January 31, 2012                [Page 3]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

1.  Introduction

The objective of Pre-Congestion Notification (PCN) [RFC5559] is to

protect the quality of service (QoS) of inelastic flows within a

Diffserv domain, in a simple, scalable, and robust fashion.  Two

mechanisms are used: admission control, to decide whether to admit or

block a new flow request, and flow termination to terminate some

existing flows during serious pre-congestion.  To achieve this, the

overall rate of PCN-traffic is metered on every link in the domain,

and PCN-packets are appropriately marked when certain configured

rates are exceeded.  These configured rates are below the rate of the

link thus providing notification to boundary nodes about overloads

before any real congestion occurs (hence "pre-congestion

notification").

[RFC5670] provides for two metering and marking functions that are

generally configured with different reference rates.  Threshold-

marking marks all PCN packets once their traffic rate on a link

exceeds the configured reference rate (PCN-threshold-rate).  Excess-

traffic-marking marks only those PCN packets that exceed the

configured reference rate (PCN-excess-rate).  The PCN-excess-rate is

typically larger than the PCN-threshold-rate [RFC5559].  Egress nodes

monitor the PCN-marks of received PCN-packets and pass information

about these PCN-marks to decision points which then decide whether to

admit new flows or terminate existing flows

[I-D.ietf-pcn-cl-edge-behaviour], [I-D.ietf-pcn-sm-edge-behaviour].

The baseline encoding defined in [RFC5696] described how two PCN

marking states (Not-marked and PCN-Marked) could be encoded into the

IP header using a single Diffserv codepoint.  It also provided an

experimental codepoint (EXP), along with guidelines for the use of

that codepoint.  Two PCN marking states are sufficient for the Single

Marking edge behaviour [I-D.ietf-pcn-sm-edge-behaviour].  However,

PCN-domains utilising the controlled load edge behaviour

[I-D.ietf-pcn-cl-edge-behaviour] require three PCN marking states.

This document extends the baseline encoding by redefining the EXP

codepoint to provide a third PCN marking state in the IP header,

still using a single Diffserv codepoint.  This encoding scheme is

therefore called the "3-in-1 PCN encoding".  It obsoletes the

baseline encoding [RFC5696], which provides only a sub-set of the

same capabilities.

The full version of this encoding requires any tunnel endpoint within

the PCN-domain to support the normal tunnelling rules defined in

[RFC6040].  There is one limited exception to this constraint where

the PCN-domain only uses the excess-traffic-marking behaviour and

where the threshold-marking behaviour is deactivated.  This is

discussed in Section 5.2.3.1.

Briscoe, et al.         Expires January 31, 2012                [Page 4]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

This document only concerns the PCN wire protocol encoding for IP

headers, whether IPv4 or IPv6.  It makes no changes or

recommendations concerning algorithms for congestion marking or

congestion response.  Other documents will define the PCN wire

protocol for other header types.  Appendix C discusses a possible

mapping between IP and MPLS.

1.1.  Requirements Language

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in [RFC2119].

1.2.  Changes in This Version (to be removed by RFC Editor)

From draft-ietf-pcn-3-in-1-encoding-06 to -07:

*  Clarified that each operator not the IETF chooses which DSCP(s)

are PCN-compatible, and made it unambiguous that only PCN-nodes

recognise that PCN-compatible DSCPs enable the 3-in-1 encoding.

*  Removed statements about the PCN working group, given RFCs are

meant to survive beyond the life of a w-g.

*  Corrected the final para of "Rationale for Different Behaviours

in Schemes with Only One Marking"

From draft-ietf-pcn-3-in-1-encoding-05 to -06:

*  Draft re-written to obsolete baseline encoding [RFC5696].

*  New section defining utilising this encoding for only one PCN-

Marking.  Added an appendix explaining an apparent

inconsistency within this section.

*  Moved (and updated) informative appendixes from [RFC5696] to

this document.  Original Appendix C was omitted as it is now

redundant.

*  Significant re-structuring of document.

From draft-ietf-pcn-3-in-1-encoding-04 to -05:

*  Draft moved to standards track as per working group

discussions.

*  Added Appendix B discussing ECN handling in the PCN-domain.

Briscoe, et al.         Expires January 31, 2012                [Page 5]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

*  Clarified that this document modifies [RFC5696].

From draft-ietf-pcn-3-in-1-encoding-03 to -04:

*  Updated document to reflect RFC6040.

*  Re-wrote introduction.

*  Re-wrote section on applicability.

*  Re-wrote section on choosing encoding scheme.

*  Updated author details.

From draft-ietf-pcn-3-in-1-encoding-02 to -03:

*  Corrected mistakes in introduction and improved overall

readability.

*  Added new terminology.

*  Rewrote a good part of Section 4 and 5 to achieve more clarity.

*  Added appendix explaining when to use which encoding scheme and

how to encode them in MPLS shim headers.

*  Added new co-author.

From draft-ietf-pcn-3-in-1-encoding-01 to -02:

*  Corrected mistake in introduction, which wrongly stated that

the threshold-traffic rate is higher than the excess-traffic

rate.  Other minor corrections.

*  Updated acks & refs.

From draft-ietf-pcn-3-in-1-encoding-00 to -01:

*  Altered the wording to make sense if

draft-ietf-tsvwg-ecn-tunnel moves to proposed standard.

*  References updated

From draft-briscoe-pcn-3-in-1-encoding-00 to

draft-ietf-pcn-3-in-1-encoding-00:

*  Filename changed to draft-ietf-pcn-3-in-1-encoding.

Briscoe, et al.         Expires January 31, 2012                [Page 6]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

*  Introduction altered to include new template description of

PCN.

*  References updated.

*  Terminology brought into line with [RFC5670].

*  Minor corrections.

2.  Definitions and Abbreviations

2.1.  Terminology

The terms PCN-domain, PCN-node, PCN-interior-node, PCN-ingress-node,

PCN-egress-node, PCN-boundary-node, PCN-traffic, PCN-packets and PCN-

marking are used as defined in [RFC5559].  The following additional

terms are defined in this document:

PCN encoding:  mapping of PCN marking states to specific codepoints

in the packet header.

PCN-compatible Diffserv codepoint:  a Diffserv codepoint indicating

packets for which the ECN field carries PCN-markings rather than

[RFC3168] markings.  Note that an operator configures PCN-nodes to

recognise PCN-compatible DSCPs, whereas the same DSCP has no PCN-

specific meaning to a node outside the PCN domain.

Threshold-marked codepoint:  a codepoint that indicates packets that

have been marked at a PCN-interior-node as a result of an

indication from the threshold-metering function [RFC5670].

Abbreviated to ThM.

Excess-traffic-marked codepoint:  a codepoint that indicates packets

that have been marked at a PCN-interior-node as a result of an

indication from the excess-traffic-metering function [RFC5670].

Abbreviated to ETM.

Not-marked codepoint:  a codepoint that indicates PCN-packets but

that are not PCN-marked.  Abbreviated to NM.

not-PCN codepoint:  a codepoint that indicates packets that are not

PCN-packets.

Briscoe, et al.         Expires January 31, 2012                [Page 7]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

2.2.  List of Abbreviations

The following abbreviations are used in this document:

o  AF = Assured Forwarding [RFC2597]

o  CE = Congestion Experienced [RFC3168]

o  CS = Class Selector [RFC2474]

o  DSCP = Diffserv codepoint

o  ECN = Explicit Congestion Notification [RFC3168]

o  ECT = ECN Capable Transport [RFC3168]

o  EF = Expedited Forwarding [RFC3246]

o  ETM = Excess-traffic-marked

o  EXP = Experimental

o  IP = Internet protocol

o  NM = Not-marked

o  PCN = Pre-Congestion Notification

o  ThM = Threshold-marked

3.  Definition of 3-in-1 PCN Encoding

The 3-in-1 PCN encoding scheme allows for two or three PCN-marking

states to be encoded within the IP header.  The full encoding is

shown in Figure 1.

+--------+----------------------------------------------------+

|        |           Codepoint in ECN field of IP header      |

|  DSCP  |               <RFC3168 codepoint name>             |

|        +--------------+-------------+-------------+---------+

|        | 00 <Not-ECT> | 10 <ECT(0)> | 01 <ECT(1)> | 11 <CE> |

+--------+--------------+-------------+-------------+---------+

| DSCP n |    Not-PCN   |      NM     |     ThM     |   ETM   |

+--------+--------------+-------------+-------------+---------+

Figure 1: 3-in-1 PCN Encoding

Briscoe, et al.         Expires January 31, 2012                [Page 8]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

A PCN-node (i.e. a node within a PCN-domain) will be configured to

recognise certain DSCPs as PCN-compatible.  Appendix A discusses the

choice of suitable DSCPs.  In Figure 1 'DSCP n' indicates such a PCN-

compatible DSCP.  Within the PCN-domain, any packet carrying a PCN-

compatible DSCP is a PCN-packet as defined in [RFC5559].

PCN-nodes MUST interpret the ECN field of a PCN-packet using the

3-in-1 PCN encoding, rather than [RFC3168].  This does not change the

behaviour for any packet with a DSCP that is not PCN-compatible, or

for any node outside a PCN-domain.  In all such cases the 3-in-1

encoding is not applicable and so by default the node will interpret

the ECN field using [RFC3168].

When using the 3-in-1 encoding, the codepoints of the ECN field have

the following meanings:

Not-PCN:  indicates a non-PCN-packet, i.e., a packet that uses a PCN-

compatible DSCP but is not subject to PCN metering and marking.

NM:  Not-marked.  Indicates a PCN-packet that has not yet been marked

by any PCN marker.

ThM:  Threshold-marked.  Indicates a PCN-packet that has been marked

by a threshold-marker [RFC5670].

ETM:  Excess-traffic-marked.  Indicates a PCN-packet that has been

marked by an excess-traffic-marker [RFC5670].

4.  Requirements for and Applicability of 3-in-1 PCN Encoding

4.1.  PCN Requirements

In accordance with the PCN architecture [RFC5559], PCN-ingress-nodes

control packets entering a PCN-domain.  Packets belonging to PCN-

controlled flows are subject to PCN-metering and -marking, and PCN-

ingress-nodes mark them as Not-marked (PCN-colouring).  Any node in

the PCN-domain may perform PCN-metering and -marking and mark PCN-

packets if needed.  There are two different metering and marking

behaviours: threshold-marking and excess-traffic-marking [RFC5670].

Some edge behaviors require only a single marking behaviour

[I-D.ietf-pcn-sm-edge-behaviour], others require both

[I-D.ietf-pcn-cl-edge-behaviour].  In the latter case, three PCN

marking states are needed: not-marked (NM) to indicate not-marked

packets, threshold-marked (ThM) to indicate packets marked by the

threshold-marker, and excess-traffic-marked (ETM) to indicate packets

marked by the excess-traffic-marker [RFC5670].  Threshold-marking and

excess-traffic-marking are configured to start marking packets at

Briscoe, et al.         Expires January 31, 2012                [Page 9]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

different load conditions, so one marking behaviour indicates more

severe pre-congestion than the other.  Therefore, a fourth PCN

marking state indicating that a packet is marked by both markers is

not needed.  However a fourth codepoint is required to indicate

packets that use a PCN-compatible DSCP but do not use PCN-marking

(the not-PCN codepoint).

In all current PCN edge behaviors that use two marking behaviours

[RFC5559], [I-D.ietf-pcn-cl-edge-behaviour], excess-traffic-marking

is configured with a larger reference rate than threshold-marking.

We take this as a rule and define excess-traffic-marked as a more

severe PCN-mark than threshold-marked.

4.2.  Requirements Imposed by Tunnelling

[RFC6040] defines rules for the encapsulation and decapsulation of

ECN markings within IP-in-IP tunnels.  The publication of RFC6040

removed the tunnelling constraints that existed when the baseline

encoding [RFC5696] was written (see section 3.3.2 of

[I-D.ietf-pcn-encoding-comparison]).

Nonetheless, there is still a problem if there are any legacy (pre-

RFC6040) decapsulating tunnel endpoints within a PCN domain.  If a

PCN node Threshold-marks the outer header of a tunnelled packet with

a Not-marked codepoint on the inner header, the legacy decapsulator

will leave the packet Not-marked after decapsulation.  The rules on

applicability in Section 4.3 below are designed to avoid this

problem.

4.3.  Applicability of 3-in-1 PCN Encoding

The 3-in-1 encoding is applicable in situations where two marking

behaviours are being used in the PCN-domain.  The 3-in-1 encoding can

also be used with only one marking behaviour, in which case one of

the codepoints MUST NOT be used throughout the PCN-domain (see

Section 5.2.3).

For the full 3-in-1 encoding to apply, any tunnel endpoints (IP-in-IP

and IPsec) within the PCN-domain MUST comply with the ECN

encapsulation and decapsulation rules set out in [RFC6040] (see

Section 4.2).  There is one exception to this rule outlined next.

If all PCN-nodes do only Excess-traffic-marking and never set the ThM codepoint, 3-in-1 encoding may be used.  Hence, 3-in-1 encoding supports pre-RFC6040 PCN domains where only Excess-traffic-marking is used, but it does not support pre-RFC6040 PCN domains where only Threshold-marking is used.

5.  Behaviour of a PCN-node to Comply with the 3-in-1 PCN Encoding

As mentioned in Section 4.3 above, all PCN-nodes MUST comply with

[RFC6040].

5.1.  PCN-ingress Node Behaviour

PCN-traffic MUST be marked with a PCN-compatible Diffserv codepoint.

To conserve DSCPs, Diffserv codepoints SHOULD be chosen that are

already defined for use with admission-controlled traffic.

Appendix A gives guidance to implementors on suitable DSCPs.

Guidelines for mixing traffic types within a PCN-domain are given in

[RFC5670].

If a packet arrives at the PCN-ingress-node that shares a PCN-

compatible DSCP and is not a PCN-packet, the PCN-ingress MUST mark it

as not-PCN.

If a PCN-packet arrives at the PCN-ingress-node, the PCN-ingress MUST

change the PCN codepoint to Not-marked.

If a PCN-packet arrives at the PCN-ingress-node with its ECN field

already set to a value other than not-ECT, then appropriate action

MUST be taken to meet the requirements of [RFC4774].  The simplest

appropriate action is to just drop such packets.  However, this is a

drastic action that an operator may feel is undesirable.  Appendix B

provides more information and summarises other alternative actions

that might be taken.

5.2.  PCN-interior Node Behaviour

5.2.1.  Behaviour Common to all PCN-interior Nodes

Interior nodes MUST NOT change not-PCN to any other codepoint.

Interior nodes MUST NOT change NM to not-PCN.

Interior nodes MUST NOT change ThM to NM or not-PCN.

Interior nodes MUST NOT change ETM to any other codepoint.

Briscoe, et al.         Expires January 31, 2012               [Page 11]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

5.2.2.  Behaviour of PCN-interior Nodes Using Two PCN-markings

If the threshold-meter function indicates a need to mark a packet,

the PCN-interior node MUST change NM to ThM.

If the excess-traffic-meter function indicates a need to mark a

packet:

o  the PCN-interior node MUST change NM to ETM;

o  the PCN-interior node MUST change ThM to ETM.

If both the threshold meter and the excess-traffic meter indicate the

need to mark a packet, the excess-traffic-marking rules MUST take

priority.

5.2.3.  Behaviour of PCN-interior Nodes Using One PCN-marking

Some PCN edge behaviours require only one PCN-marking within the PCN-

domain.  The Single Marking edge behaviour

[I-D.ietf-pcn-sm-edge-behaviour] requires PCN-interior nodes to mark

packets using the excess-traffic-meter function [RFC5670].  It is

possible that future schemes may require only the threshold-meter

function.  Observant readers may spot an apparent inconsistency

between the two following cases.  Appendix D explains the rationale

behind this inconsistency.

5.2.3.1.  Marking Using only the Excess-traffic-meter Function

The threshold-traffic-meter function SHOULD be disabled and MUST NOT

trigger any packet marking.

The PCN-interior node SHOULD raise a management alarm if it receives

a ThM packet, but the frequency of such alarms SHOULD be limited.

If the excess-traffic-meter function indicates a need to mark the

packet:

o  the PCN-interior node MUST change NM to ETM;

o  the PCN-interior node MUST change ThM to ETM.  It SHOULD also

raise an alarm as above.

5.2.3.2.  Marking using only the Threshold-meter Function

The excess-traffic-meter function SHOULD be disabled and MUST NOT

trigger any packet marking.

Briscoe, et al.         Expires January 31, 2012               [Page 12]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

The PCN-interior node SHOULD raise a management alarm if it receives

an ETM packet, but the frequency of such alarms SHOULD be limited.

If the threshold-meter function indicates a need to mark the packet:

o  the PCN-interior node MUST change NM to ThM;

o  the PCN-interior node MUST NOT change ETM to any other codepoint.

It SHOULD raise an alarm as above if it encounters an ETM packet.

5.3.  PCN-egress Node Behaviour

A PCN-egress-node SHOULD set the not-PCN (00) codepoint on all

packets it forwards out of the PCN-domain.

The only exception to this is if the PCN-egress-node is certain that

revealing other codepoints outside the PCN-domain won't contravene

the guidance given in [RFC4774].  For instance, if the PCN-ingress-

node has explicitly informed the PCN-egress-node that this flow is

ECN-capable, then it might be safe to expose other codepoints.

Appendix B gives details of how such schemes might work, but such

schemes are currently only tentative ideas.

If the PCN-domain is configured to use only excess-traffic marking,

the PCN-egress node MUST treat ThM as ETM and, if only threshold-

marking is used, it should treat ETM as ThM.  However it SHOULD raise

a management alarm in either instance since this means there is some

misconfiguration in the PCN-domain.

6.  Backward Compatibility

6.1.  Backward Compatibility with ECN

BCP 124 [RFC4774] gives guidelines for specifying alternative

semantics for the ECN field.  It sets out a number of factors to be

taken into consideration.  It also suggests various techniques to

allow the co-existence of default ECN and alternative ECN semantics.

The encoding specified in this document uses one of those techniques;

it defines PCN-compatible Diffserv codepoints as no longer supporting

the default ECN semantics within a PCN domain.  As such, this document is compatible with

BCP 124.

On its own, the 3-in-1 encoding cannot support both ECN marking end-

to-end (e2e) and PCN-marking within a PCN-domain.  Appendix B

discusses possible ways to do this, e.g. by carrying e2e ECN across a

PCN-domain within the inner header of an IP-in-IP tunnel.  Although

Appendix B recommends various approaches over others, it is merely

Briscoe, et al.         Expires January 31, 2012               [Page 13]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

informative and all such schemes are beyond the normative scope of

this document.

In any PCN deployment, traffic can only enter the PCN-domain through

PCN-ingress-nodes and leave through PCN-egress-nodes.  PCN-ingress-

nodes ensure that any packets entering the PCN-domain have the ECN

field in their outermost IP header set to the appropriate PCN

codepoint.  PCN-egress-nodes then guarantee that the ECN field of any

packet leaving the PCN-domain has appropriate ECN semantics.  This

prevents unintended leakage of ECN marks into or out of the PCN-

domain, and thus reduces backward-compatibility issues.

6.2.  Backward Compatibility with the Baseline Encoding

A PCN node implemented to use the obsoleted baseline encoding could

conceivably have been configured so that the Threshold-meter function

marked what is now defined as the ETM codepoint in the 3-in-1

encoding.  However, there is no known deployment of such an

implementation and no reason to believe that such an implementation

would ever have been built.  Therefore, it seems safe to ignore this

issue.

7.  IANA Considerations

This memo includes no request to IANA.

Note to RFC Editor: this section may be removed on publication as an

RFC.

8.  Security Considerations

PCN-marking only carries a meaning within the confines of a PCN-

domain.  This encoding document is intended to stand independently of

the architecture used to determine how specific packets are

authorised to be PCN-marked, which will be described in separate

documents on PCN-boundary-node behaviour.

This document assumes the PCN-domain to be entirely under the control

of a single operator, or a set of operators who trust each other.

However, future extensions to PCN might include inter-domain versions

where trust cannot be assumed between domains.  If such schemes are

proposed, they must ensure that they can operate securely despite the

lack of trust.  However, such considerations are beyond the scope of

this document.

One potential security concern is the injection of spurious PCN-marks

Briscoe, et al.         Expires January 31, 2012               [Page 14]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

into the PCN-domain.  However, these can only enter the domain if a

PCN-ingress-node is misconfigured.  The precise impact of any such

misconfiguration will depend on which of the proposed PCN-boundary-

node behaviours is used, but in general spurious marks will lead to

admitting fewer flows into the domain or potentially terminating too

many flows.  In either case, good management should be able to

quickly spot the problem since the overall utilisation of the domain

will rapidly fall.

9.  Conclusions

The 3-in-1 PCN encoding uses a PCN-compatible DSCP and the ECN field

to encode PCN-marks.  One codepoint allows non-PCN traffic to be

carried with the same PCN-compatible DSCP and three other codepoints

support three PCN marking states with different levels of severity.

In general, the use of this PCN encoding scheme presupposes that any

tunnel endpoints within the PCN-domain comply with [RFC6040].

10.  Acknowledgements

Many thanks to Phil Eardley for providing extensive feedback,

critcism and advice.  Thanks also to Teco Boot, Kwok Ho Chan,

Ruediger Geib, Georgios Karaginannis and everyone else who has

commented on the document.

11.  Comments Solicited

To be removed by RFC Editor: Comments and questions are encouraged

and very welcome.  They can be addressed to the IETF Congestion and

Pre-Congestion working group mailing list <pcn@ietf.org>, and/or to

the authors.

12.  References

12.1.  Normative References

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119, March 1997.

[RFC2474]  Nichols, K., Blake, S., Baker, F., and D. Black,

"Definition of the Differentiated Services Field (DS

Field) in the IPv4 and IPv6 Headers", RFC 2474,

December 1998.

Briscoe, et al.         Expires January 31, 2012               [Page 15]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

[RFC3168]  Ramakrishnan, K., Floyd, S., and D. Black, "The Addition

of Explicit Congestion Notification (ECN) to IP",

RFC 3168, September 2001.

[RFC5559]  Eardley, P., "Pre-Congestion Notification (PCN)

Architecture", RFC 5559, June 2009.

[RFC5670]  Eardley, P., "Metering and Marking Behaviour of PCN-

Nodes", RFC 5670, November 2009.

[RFC6040]  Briscoe, B., "Tunnelling of Explicit Congestion

Notification", RFC 6040, November 2010.

12.2.  Informative References

[I-D.ietf-pcn-cl-edge-behaviour]

Charny, A., Huang, F., Karagiannis, G., Menth, M., and T.

Taylor, "PCN Boundary Node Behaviour for the Controlled

Load (CL) Mode of Operation",

draft-ietf-pcn-cl-edge-behaviour-09 (work in progress),

June 2011.

[I-D.ietf-pcn-encoding-comparison]

Karagiannis, G., Chan, K., Moncaster, T., Menth, M.,

Eardley, P., and B. Briscoe, "Overview of Pre-Congestion

Notification Encoding",

draft-ietf-pcn-encoding-comparison-06 (work in progress),

June 2011.

[I-D.ietf-pcn-sm-edge-behaviour]

Charny, A., Karagiannis, G., Menth, M., and T. Taylor,

"PCN Boundary Node Behaviour for the Single Marking (SM)

Mode of Operation", draft-ietf-pcn-sm-edge-behaviour-06

(work in progress), June 2011.

[RFC2597]  Heinanen, J., Baker, F., Weiss, W., and J. Wroclawski,

"Assured Forwarding PHB Group", RFC 2597, June 1999.

[RFC3246]  Davie, B., Charny, A., Bennet, J., Benson, K., Le Boudec,

J., Courtney, W., Davari, S., Firoiu, V., and D.

Stiliadis, "An Expedited Forwarding PHB (Per-Hop

Behavior)", RFC 3246, March 2002.

[RFC3540]  Spring, N., Wetherall, D., and D. Ely, "Robust Explicit

Congestion Notification (ECN) Signaling with Nonces",

RFC 3540, June 2003.

[RFC4594]  Babiarz, J., Chan, K., and F. Baker, "Configuration

Briscoe, et al.         Expires January 31, 2012               [Page 16]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

Guidelines for DiffServ Service Classes", RFC 4594,

August 2006.

[RFC4774]  Floyd, S., "Specifying Alternate Semantics for the

Explicit Congestion Notification (ECN) Field", BCP 124,

RFC 4774, November 2006.

[RFC5127]  Chan, K., Babiarz, J., and F. Baker, "Aggregation of

DiffServ Service Classes", RFC 5127, February 2008.

[RFC5129]  Davie, B., Briscoe, B., and J. Tay, "Explicit Congestion

Marking in MPLS", RFC 5129, January 2008.

[RFC5462]  Andersson, L. and R. Asati, "Multiprotocol Label Switching

(MPLS) Label Stack Entry: "EXP" Field Renamed to "Traffic

Class" Field", RFC 5462, February 2009.

[RFC5696]  Moncaster, T., Briscoe, B., and M. Menth, "Baseline

Encoding and Transport of Pre-Congestion Information",

RFC 5696, November 2009.

[RFC5865]  Baker, F., Polk, J., and M. Dolly, "A Differentiated

Services Code Point (DSCP) for Capacity-Admitted Traffic",

RFC 5865, May 2010.

Appendix A.  Choice of Suitable DSCPs

This appendix is informative, not normative.

A single DSCP has not been defined for use with PCN for several

reasons.  Firstly, the PCN mechanism is applicable to a variety of

different traffic classes.  Secondly, Standards Track DSCPs are in

increasingly short supply.  Thirdly, PCN is not a scheduling

behaviour -- rather, it should be seen as being a marking behaviour

similar to ECN but intended for inelastic traffic.  The choice of

which DSCP is most suitable for a given PCN-domain is dependent on

the nature of the traffic entering that domain and the link rates of

all the links making up that domain.  In PCN-domains with sufficient

aggregation, the appropriate DSCPs would currently be those for the

Real-Time Treatment Aggregate [RFC5127].  It is suggested that

admission control could be used for the following service classes

(defined in [RFC4594] unless otherwise stated):

o  Telephony (EF)

o  Real-time interactive (CS4)

Briscoe, et al.         Expires January 31, 2012               [Page 17]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

o  Broadcast Video (CS3)

o  Multimedia Conferencing (AF4)

o  the VOICE-ADMIT codepoint defined in [RFC5865].

CS5 is excluded from this list since PCN is not expected to be

applied to signalling traffic.

PCN-marking is intended to provide a scalable admission-control

mechanism for traffic with a high degree of statistical multiplexing.

PCN-marking would therefore be appropriate to apply to traffic in the

above classes, but only within a PCN-domain containing sufficiently

aggregated traffic.  In such cases, the above service classes may

well all be subject to a single forwarding treatment (treatment

aggregate [RFC5127]).  However, this does not imply all such IP

traffic would necessarily be identified by one DSCP -- each service

class might keep a distinct DSCP within the highly aggregated region

[RFC5127].

Additional service classes may be defined for which admission control

is appropriate, whether through some future standards action or

through local use by certain operators, e.g., the Multimedia

Streaming service class (AF3).  This document does not preclude the

use of PCN in more cases than those listed above.

Note: The above discussion is informative not normative, as operators

are ultimately free to decide whether to use admission control for

certain service classes and whether to use PCN as their mechanism of

choice.

Appendix B.  Co-existence of ECN and PCN

This appendix is informative, not normative.

The PCN encoding described in this document re-uses the bits of the

ECN field in the IP header.  Consequently, this disables ECN within

the PCN domain.  Appendix B of [RFC5696] (obsoleted) included advice

on handling ECN traffic within a PCN-domain.  This appendix

reiterates and clarifies that advice.

For the purposes of this appendix we define two forms of traffic that

might arrive at a PCN-ingress node.  These are admission-controlled

traffic and non-admission-controlled traffic.

Admission-controlled traffic will be re-marked to a PCN-compatible

DSCP by the PCN-ingress node.  Two mechanisms can be used to identify

Briscoe, et al.         Expires January 31, 2012               [Page 18]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

such traffic:

a.  Flow signalling associates a filterspec with a need for admission

control (e.g. through RSVP or some equivalent message, e.g. from

a SIP server to the ingress); the PCN-ingress re-marks

traffic matching that filterspec to a PCN-compatible DSCP

.

b.  Traffic arrives with a DSCP that implies it requires admission

control such as VOICE-ADMIT [RFC5865] or Interactive Real-Time,

Broadcast TV when used for video on demand, and multimedia

conferencing [RFC4594][RFC5865] (see Appendix A).

All other traffic can be thought of as non-admission-controlled (and

therefore outside the scope of PCN).  However such traffic may still

need to share the same DSCP as the admission-controlled traffic.

This may be due to policy (for instance if it is high priority voice

traffic), or may be because there is a shortage of local DSCPs.

ECN [RFC3168] is an end-to-end congestion notification mechanism.  As

such it is possible that some traffic entering the PCN-domain may

also be ECN capable.

Unless specified otherwise, for any of the cases in the list below,

an IP-in-IP tunnel can be used to preserve ECN markings across the

PCN domain.  The tunnelling action should be applied wholly outside

the PCN-domain as illustrated in the following figure:

,  .  .  .  .  .  PCN-domain  .  .  .  .  .  .

.   ,--------.                   ,--------.    .

.   _|  PCN-   |___________________|  PCN-  |_   .

.  / | ingress |                   | egress | \  .

.|  '---------'                   '--------'  |.

| .  .  .  .  .  .  .  .  .  .  .  .  .  .  .|

,--------.                                     ,--------.

_____| Tunnel  |                                     | Tunnel |____

| Ingress | - - ECN preserved inside tunnel - - | Egress |

'---------'                                     '--------'

Figure 2: Separation of tunneling and PCN actions.

There are three cases for how e2e ECN traffic may wish to be treated

while crossing a PCN domain:

a) Traffic that does not require admission control (e.g. traffic that does not match flow signaling being used for admission control):

*  Does not carry a PCN-compatible DSCP: no action required.

Briscoe, et al.         Expires January 31, 2012               [Page 19]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

*  Arrives carrying a DSCP that clashes with a PCN-

compatible DSCP: there are two options:

1.  The ingress maps the DSCP to a local DSCP with the same

scheduling PHB as the original DSCP, and the egress re-maps

it to the original PCN-compatible DSCP.

2.  The ingress tunnels the traffic, setting not-PCN in the

outer header; note that this turns off ECN for this traffic

within the PCN domain.

The first option is recommended unless the operator is short of

local DSCPs.

b) Traffic that requires admission control:  there are two options.

*  The PCN-ingress places this traffic in a tunnel with a PCN-

compatible DSCP in the outer header.

*  The PCN-ingress drops CE-marked packets and the PCN-egress

zeroes the ECN field of all PCN packets.

The second option is emphatically not recommended, unless perhaps

as a last resort if tunnelling is not possible for some

insurmountable reason.

c) Traffic that requires admission control and asks to see PCN marks:  note that  this

scheme is currently only a tentative idea.

For real-time data generated by an adaptive codec, schemes have

been suggested where PCN marks may be leaked out of the PCN-domain

so that end hosts can drop to a lower data rate, thus deferring

the need for admission control.  Currently such schemes require

further study and the following is for guidance only.

The PCN-ingress needs to tunnel the traffic as in Figure 2, taking

care to comply with [RFC6040].  In this case the PCN-egress should

not zero the ECN field contrary to the recommendation in Section 5.3, and then the [RFC6040] tunnel egress will

preserve any PCN-marking.  Note that a PCN interior node may turn

ECT(0) into ECT(1), which would not be compatible with the

(currently experimental) ECN nonce [RFC3540].

Appendix C.  Example Mapping between Encoding of PCN-Marks in IP and in

MPLS Shim Headers

This appendix is informative not normative.

Briscoe, et al.         Expires January 31, 2012               [Page 20]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

The 6 bits of the DS field in the IP header provide for 64

codepoints.  When encapsulating IP traffic in MPLS, it is useful to

make the DS field information accessible in the MPLS header.

However, the MPLS shim header has only a 3-bit traffic class (TC)

field [RFC5462] providing for 8 codepoints.  The operator has the

freedom to define a site-local mapping of the 64 codepoints of the DS

field onto the 8 codepoints in the TC field.

[RFC5129] describes how ECN markings in the IP header can also be

mapped to codepoints in the MPLS TC field.  Appendix A of [RFC5129]

gives an informative description of how to support PCN in MPLS by

extending the way MPLS supports ECN.  But [RFC5129] was written while

PCN specifications were in early draft stages.  The following

provides a clearer example of a mapping between PCN in IP and in MPLS

using the PCN terminology and concepts that have since been

specified.

To support PCN in a MPLS domain, a PCN-compatible DSCP ('DSCP n')

needs codepoints to be provided in the TC field for all the PCN-marks

used.  That means, when for instance only excess-traffic-marking is

used for PCN purposes, the operator needs to define a site-local

mapping to two codepoints in the MPLS TC field for IP headers with:

o  DSCP n and ECT(0)

o  DSCP n and CE

If both excess-traffic-marking and threshold-marking are used, the

operator needs to define a site-local mapping to codepoints in the

MPLS TC field for IP headers with all three of the 3-in-1 codepoints:

o  DSCP n and ECT(0)

o  DSCP n and ECT(1)

o  DSCP n and CE

In either case, if the operator wishes to support the same Diffserv

PHB but without PCN marking, it will also be necessary to define a

site-local mapping to an MPLS TC codepoint for IP headers marked

with:

o  DSCP n and Not-ECT

Clearly, given so few TC codepoints are available, it may be

necessary to compromise by merging together some capabilities.

Briscoe, et al.         Expires January 31, 2012               [Page 21]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

Appendix D.  Rationale for Discrepancy Between the Schemes using One

PCN-Marking

Readers may notice an apparent discrepancy between the two behaviours

in Section 5.2.3.1 and Section 5.2.3.2.  With only excess-traffic

marking enabled, an unexpected ThM packet can be re-marked to ETM.

However, with only threshold-marking, an unexpected ETM packet cannot

be re-marked to ThM.

This apparent inconsistency is deliberate, for two reasons:

o  If only one type of marking function is meant to be used

throughout the PCN-domain but the other type unexpectedly appears

on some packets, it is safest to assume that some link is trying

to signal that it is pre-congested, but that it is somehow using

the wrong signal.  This only needs to be corrected if the

behaviour of other nodes depends on the marking a packet arrives

with.  In [RFC5670], the excess-traffic-metering behaviour depends

on the markings on arriving packets, whereas threshold-metering

does not.  Therefore, if ThM should not be present, it seems safe

to allow it to be re-marked to ETM, but if ETM should not be

present there is no need to re-mark it to ThM.

o  The behaviour with only threshold marking keeps to the rule that

ETM is more severe and must never be changed to ThM even though

ETM is not a valid marking in this case.  Otherwise

implementations would have to allow operators to configure an

exception to this rule, which would not be safe practice.

Authors' Addresses

Bob Briscoe

BT

B54/77, Adastral Park

Martlesham Heath

Ipswich  IP5 3RE

UK

Phone: +44 1473 645196

Email: bob.briscoe@bt.com

URI:   http://bobbriscoe.net/

Briscoe, et al.         Expires January 31, 2012               [Page 22]

Internet-Draft             3-in-1 PCN Encoding                 July 2011

Toby Moncaster

Moncaster Internet Consulting

Dukes

Layer Marney

Colchester  CO5 9UZ

UK

Phone: +44 7764 185416

Email: toby@moncaster.com

URI:   http://www.moncaster.com/

Michael Menth

University of Tuebingen

Sand 13

Tuebingen  72076

Germany

Phone: +49 7071 2970505

Email: menth@informatik.uni-tuebingen.de

Briscoe, et al.         Expires January 31, 2012               [Page 23]